# TOKO_ALGORITHMS: Software Packages Division

# SMART PAD Firmware

Runs on: Microcontroller in each SMART PAD

Inputs

- Power and I²C bus from BASE UNIT

- Signals from 2 × FSR402 pressure sensors

Outputs (sent via I²C)

- Touch event packets: {Pad_ID, Intensity1, Intensity2, Duration, Timestamp}

- Configuration data (when requested): {Pad_ID, LED_count (1–6), Vibrator_present, Calibration_values}

Main Responsibilities

- Low-power operation (deep sleep + wake on timer or touch threshold)

- Read FSR sensors → detect and classify touches

- Execute received actuator commands (turn on/off LEDs and vibrator)

- Respond to configuration read requests

## SMART PAD Firmware – High-Level Algorithm

- Power applied to SMART PAD (via magnetic pogo pins from BASE UNIT)

- MCU boots and initialises I²C slave mode

- Calibrate baselines: read both FSR sensors 20 times with no touch → calculate average baseline1 and baseline2 → store

- Enter main low-power loop

- Every 100 ms: wake from sleep → read raw values from both FSR sensors

- Subtract baselines → calculate current force1 and force2

- If no touch active AND (force1 or force2 > TOUCH_THRESHOLD): → Start new touch → Record start timestamp → Set peak_intensity to current max force

- If touch active: → Update peak_intensity if current force is higher → Continue measuring duration

- If touch active AND (force1 and force2 < RELEASE_THRESHOLD for 100 ms): → End touch → Calculate total duration → Create Touch Event packet (Pad_ID, peak_intensity, duration, sensor balance) → Send packet via I²C to BASE UNIT Driver → Reset touch state

- At any time: listen for incoming I²C commands from CONTROL UNIT

- If command received: → If SET_LED → turn on/off or pattern specific LEDs → If SET_VIBRATOR → turn on/off vibrator with specified intensity and duration → If READ_CONFIG → send back stored configuration (Pad_ID, LED_count, vibrator_present, etc.)

- After processing: return to sleep until next 100 ms wake or command interrupt

- Repeat steps 5–12 continuously

# BASE UNIT Driver

Runs on: Raspberry Pi in CONTROL UNIT

Inputs

- Physical I²C lines from BASE UNIT cable

Outputs

- Processed data to CONTROL UNIT Core: pad presence, configuration, touch events

- Actuator commands sent down to specific pads/slots

Main Responsibilities

- Scan all 12 slots for pad presence (periodic or interrupt)

- Handle hot-plug (detect connect/disconnect)

- Assign/manage I²C addresses if needed

- Read configuration from each connected pad at startup or plug-in

- Forward touch events to Core

- Route actuator commands from Core to correct pad/slot

## BASE UNIT Driver – High-Level Algorithm

- CONTROL UNIT (Raspberry Pi) boots and starts the BASE UNIT Driver software

- Initialise I²C master interface (connected to BASE UNIT cable)

- Perform full system scan: for each of the 12 slots (1 to 12) → Temporarily select or address the slot (if multiplexed) or use general call → Send READ_CONFIG command → Wait for response from SMART PAD (if any) → If response received → store Pad_ID, LED_count, vibrator_present, etc. for that slot → If no response → mark slot as empty

- Build and store global Pad Map (slot 1–12 → details or empty)

- Send initial Pad Map to CONTROL Core

- Enter main monitoring loop

- Every 500 ms: quick re-scan all slots for changes (hot-plug detection) → If a slot changed from empty to occupied → read new config → update Pad Map → notify Core (“new pad plugged”) → If a slot changed from occupied to empty → update Pad Map → notify Core (“pad removed”)

- Continuously listen for incoming I²C packets from any SMART PAD

- When Touch Event packet arrives: → Identify which slot sent it → Add slot number to packet → Forward complete event immediately to CONTROL Core

- When CONTROL Core sends an actuator command (LED or vibration for specific slot): → Route the command via I²C to the correct slot/pad → Wait for optional acknowledgment (if implemented)

- If app requests current pad configuration (via Core): provide latest Pad Map

- Repeat steps 7–11 continuously until shutdown

- On shutdown command from Core: cleanly stop I²C and release resources

# CONTROL UNIT Core Software

Runs on: Raspberry Pi in CONTROL UNIT

Inputs

- From BASE UNIT Driver: pad map, hot-plug events, touch events

- From TOKOApp (via BT/Wi-Fi): commands (set level/mode, upload audio, start/stop, etc.)

Outputs

- To BASE UNIT Driver: actuator commands (LED patterns, vibration per pad)

- To Audio & Storage Manager: playback triggers (sound ID or custom file)

- To TOKOApp: live events, current state, acknowledgments, stats

- To Audio & Storage Manager: log data for storage

Main Responsibilities

- Maintain live map of all pads (slot → Pad_ID → capabilities)

- Run Game Engine: process touches → apply current level rules → decide rewards/sequences

- Manage play modes and progressive levels

- Generate light chases, attention sequences, multi-pad patterns

- Handle real-time communication with app

## CONTROL UNIT Core Software – High-Level Algorithm

- CONTROL UNIT (Raspberry Pi) boots and starts the Core Software

- Wait for BASE UNIT Driver to complete initial full scan and provide first Pad Map

- Receive and store current Pad Map (slot 1–12 → Pad_ID, LED_count, vibrator, theme, etc.)

- Load default or last-used play settings (level, mode, volume, custom audio assignments) from storage

- If TOKOApp is connected via Bluetooth/Wi-Fi → send current Pad Map and system state to app

- Enter main processing loop

- Continuously listen for three types of inputs: a. Touch Events from BASE UNIT Driver b. Hot-plug notifications (pad added/removed) from Driver c. Commands from TOKOApp (set level, change mode, upload custom audio, etc.)

- When Hot-plug notification received: → Update internal Pad Map → Notify connected TOKOApp of change

- When command received from TOKOApp: → Update settings (level, mode, volume, etc.) → Store new custom audio assignments if provided → Send acknowledgment back to app

- When Touch Event received (slot + intensity + duration): → Look up Pad_ID and capabilities for that slot → Apply current play level/mode rules (e.g., Level 1: only reward single touch; higher levels allow sequences or multi-pad) → Decide response: reward, ignore, or part of sequence → If reward needed → generate actuator commands (LED pattern, vibration) for one or more pads → Send actuator commands to BASE UNIT Driver → Trigger Audio & Storage Manager to play appropriate sound (default or custom voice for this pad/action) → Log full event (timestamp, slot, Pad_ID, intensity, duration, response type) → If TOKOApp connected → send live event update

- Periodically (every few seconds when no touch): → If in attention-seeking mode → generate proactive light sequence (traveling lights, pulsing) to attract child → Send those actuator commands via Driver

- When statistics or session data requested by TOKOApp: → Retrieve logged events from Audio & Storage Manager → Calculate summaries (daily touches, favorite pads, progress) → Send to app

- Repeat steps 7–12 continuously until shutdown

- On shutdown command: save current settings, stop all actuators and audio, cleanly exit

# Audio & Storage Manager

Runs on: Raspberry Pi in CONTROL UNIT (same device as Core)

Inputs

- Playback triggers from Core (e.g., “play reward_3”, “play custom_elepant_pad”)

- Custom audio files + metadata from TOKOApp

Outputs

- Audio signal to built-in speaker

- Confirmation to Core (playback started/finished)

- Stored files and logs on SD card/flash

Main Responsibilities

- Store default sounds and custom recordings

- Replace/assign custom audio to specific pads or reward actions

- Play audio files on demand without blocking main loop

- Log all events (touches, responses, timestamps) for statistics

## Audio & Storage Manager – High-Level Algorithm

- CONTROL UNIT boots and starts the Audio & Storage Manager (runs alongside Core)

- Initialise audio hardware (speaker, amplifier) and check volume level

- Load default sound library from internal storage (rewards, music, prompts like “Find the elephant”)

- Check for custom audio files on SD card/flash → load list of available custom recordings and their assignments (e.g., “grandma_reward_1.mp3” assigned to Pad_ID 125 correct touch)

- Build internal Audio Map: sound ID → file path + assigned trigger (pad, reward type, etc.)

- Send “ready” signal to CONTROL Core

- Enter main waiting loop

- Continuously listen for playback triggers from CONTROL Core

- When trigger received: → Example: “play reward_3” or “play custom for Pad_ID 125” or “play attention melody” → Look up file path in Audio Map → Load and play the audio file through speaker (non-blocking if possible) → Adjust volume according to current setting → Send “playback started” and “playback finished” signals back to Core (so Core can synchronise lights)

- When new custom audio arrives from TOKOApp (via Core): → Receive audio file + metadata (name, assigned pad/reward) → Save file securely to storage → Update Audio Map with new assignment → Overwrite or replace old custom sound if requested → Confirm success to Core → Core notifies app

- When log request received from Core: → Append event details to daily log file (timestamp, touch data, response, sound played) → Keep logs organised by date/session

- When statistics request received from Core (for app): → Read relevant log files → Calculate summaries (total touches, touches per pad, session duration, progress) → Return structured data to Core

- Periodically (once per day or on shutdown): → Clean old logs if storage is low → Backup critical settings

- Repeat steps 8–13 continuously until shutdown

- On shutdown: stop any playing audio, save current Audio Map and settings

# TOKOApp (Parent App)

Runs on: Parent’s smartphone (iOS/Android)

Inputs

- From CONTROL UNIT (BT/Wi-Fi): live events, pad map, current state, statistics

- From cloud: pending grandparent recordings (from TOKOplay)

Outputs

- To CONTROL UNIT: commands (set level, start mode, upload custom audio, etc.)

- To cloud: approval/rejection of grandparent recordings

Main Responsibilities

- Basic Controls & Levels section

- Customization section (record and assign messages)

- Play Modes section (select mode, live monitoring)

- Statistics section (charts and progress – requires sign-up)

## TOKOApp (Parent App) – High-Level Algorithm

- Parent opens TOKOApp on smartphone (iOS or Android)

- App checks for saved CONTROL UNIT pairing (Bluetooth ID or Wi-Fi credentials)

- App scans for nearby CONTROL UNIT via Bluetooth (primary) or local Wi-Fi

- If found → attempt connection → authenticate with unique device ID

- On successful connection: → Request current system state (Pad Map, current level/mode, volume) → Request recent statistics summary → Display home screen with live status

- Parent navigates to one of the 4 main sections

- Basic Controls & Levels section: → Parent selects new level (e.g., Level 3) or mode (free play, follow-light) → App sends command to CONTROL UNIT → Wait for acknowledgment → update UI → Parent can turn system on/off or adjust volume → same send/acknowledge flow

- Customization section: → Parent taps “Record message” → records voice using phone microphone → Parent assigns recording to specific pad (e.g., Pad_ID 125) or reward type → App sends audio file + metadata to CONTROL UNIT → Wait for confirmation → show “saved” → Parent can also browse and play/delete existing custom recordings

- Play Modes section: → Parent selects a game mode → app sends mode command to CONTROL UNIT → App starts receiving live touch events and actuator updates from CONTROL UNIT → Display real-time visualisation (lights glowing on virtual rail, sounds playing) → Parent can stop or pause session at any time

- Statistics section (requires sign-up/login): → App requests full log data or summaries from CONTROL UNIT → If online → sync with cloud for backups or multi-device access → Display charts: daily touches, touches per pad, progress over time, favourite themes

- Cloud integration for grandparent recordings: → App checks cloud for pending recordings from TOKOplay → Show list → parent listens → approve or reject → On approve → download recording → send to CONTROL UNIT as custom audio

- Background handling: → If connection lost → show “disconnected” → auto-reconnect when in range → Keep local cache of last known Pad Map and settings

- App runs continuously in foreground or background (with notifications for new grandparent recordings) until closed

- On app close or phone shutdown: disconnect cleanly from CONTROL UNIT

# TOKOplay App (Grandparent App)

Runs on: Grandparent’s smartphone

Inputs

- Parent approval status from cloud

Outputs

- Recorded audio files + metadata → cloud (for parent approval)

Main Responsibilities

- Simple interface to record voice messages

- Link to specific CONTROL UNIT (via parent-shared ID or QR)

- Send recordings to cloud for parent review

## TOKOplay App  – High-Level Algorithm

- Grandparent opens TOKOplay App on their smartphone (simple, minimal interface)

- App checks for previously paired CONTROL UNIT (via parent-shared unique ID or QR code)

- If not paired → show instructions: “Ask parent for the TOKOTouch ID or QR code”

- Grandparent scans QR or enters ID provided by parent → app stores the pairing

- App shows main screen: large “Record New Message” button + list of previous recordings

- Grandparent taps “Record New Message”

- App shows simple record screen: animal/theme selection (e.g., elephant, car, general reward) if multiple options

- Grandparent presses record button → speaks message (e.g., “Well done, my little one!”) → stops recording

- App plays back recording for review

- Grandparent adds optional text note (e.g., “For correct elephant touch”)

- Grandparent confirms and taps “Send to Parent”

- App uploads recording file + metadata (linked CONTROL UNIT ID, theme/pad suggestion, grandparent name) to Cloud

- App shows “Sent – waiting for parent approval”

- Cloud notifies parent’s TOKOApp of new pending recording

- If parent approves (in TOKOApp): Cloud sends approval → TOKOplay optionally shows “Approved and added!” notification (if app is open)

- If parent rejects: similar notification “Parent asked to re-record”

- Grandparent can view history of sent recordings and their status

- App remains simple: no live monitoring, no statistics, no controls – only recording and sending

- Background: app can receive push notifications for status updates

- On app close: save pairing and history locally

# High-Level Algorithm Overview & Connections

## Global Parameters Shared Across All Packages

These are the key values that connect everything (stored in CONTROL UNIT and synced where needed):

- Pad Map: Slot 1–12 → Pad_ID (e.g., 125 = elephant) → LED_count (e.g., 5) → Vibrator (yes/no)

- Current Play Level/Mode: Set by TOKOApp → Core (e.g., Level 1 = reward single touch only)

- Touch Thresholds: Configurable via App (e.g., min intensity = 50)

- Custom Audio Assignments: App → Storage (e.g., "Pad 125 correct touch → play grandma voice")

- Log Data: Touch events + responses → Storage → App stats

## High-Level Flow Example (Child touches pad)

- SMART PAD: Detects touch → sends packet via I²C

- BASE UNIT Driver: Receives → forwards to Core

- Core: Checks current level → decides reward (lights + sound)

- Core → Driver: Send LED/vibration commands to pad

- Core → Audio: Play reward sound (default or custom)

- Core → App (if connected): Send live event + update stats

- Storage: Save event for later stats

## High Level Algorithm

- Grandparent opens TOKOplay App

- Grandparent records a voice message (e.g., “Great job finding the elephant!”)

- TOKOplay sends recording + metadata to Cloud

- Parent receives notification in TOKOApp

- Parent reviews and approves/rejects the recording in TOKOApp

- Approved recording downloads from Cloud to TOKOApp → sent to CONTROL UNIT via Bluetooth/Wi-Fi

- CONTROL UNIT stores custom recording (Audio & Storage Manager) and assigns it to a pad or reward

- Parent uses TOKOApp to set play level, mode, or basic controls → sends command to CONTROL UNIT

- CONTROL UNIT powers BASE UNIT and SMART PADs via cable

- On startup or plug-in: BASE UNIT Driver scans all 12 slots → each SMART PAD reports its Pad_ID and configuration

- CONTROL Core builds live pad map (which slot has which animal/theme, how many LEDs, etc.)

- Child touches a SMART PAD

- SMART PAD detects pressure on FSR sensors → measures intensity & duration → sends Touch Event via I²C

- BASE UNIT Driver receives event → forwards to CONTROL Core

- CONTROL Core applies current level/mode rules → decides response (reward or attention sequence)

- CONTROL Core sends LED and vibration commands back through BASE UNIT Driver to specific SMART PAD(s)

- CONTROL Core triggers Audio & Storage Manager to play sound (default or custom voice)

- CONTROL Core logs the full event (touch + response + timestamp)

- If TOKOApp is connected → CONTROL UNIT sends live update and updated statistics to phone

- Parent sees real-time activity or later views charts in Statistics section of TOKOApp

# TOKO_ALGORITHMS – High-Level Database Structure

(For CONTROL UNIT Local Storage + Optional Cloud Sync) (Numbered Sections + Tables – Copies perfectly into Word)

## General Principles

- Primary database runs locally on CONTROL UNIT (Raspberry Pi SD card) using SQLite (simple, file-based, no server needed)

- All tables include auto-increment ID and timestamp fields for easy querying

- Data kept for 90–180 days locally, older data auto-archived or deleted

- Optional cloud sync (for signed-in users) mirrors key tables anonymously

- Future-proof: includes fields for advanced touch classification (even if not used yet)

## Main Database Tables

### Table 1: Pads (Static + Dynamic Configuration)

| Field | Type | Description | Example |
| --- | --- | --- | --- |
| pad_id | INTEGER PK | Unique Pad_ID (e.g., 125 = elephant) | 125 |
| theme_name | TEXT | Human name | "Elephant" |
| led_count | INTEGER | Number of RGB LEDs | 5 |
| vibrator_present | BOOLEAN | Yes/No | 1 |
| calibration_data | TEXT (JSON) | Baseline offsets for 2 FSRs | {"fsr1": 102, "fsr2": 98} |
| default_sound_id | INTEGER | Linked default system sound | 12 |
| custom_sound_id | INTEGER | Linked custom recording (nullable) | 45 or NULL |

### Table 2: Current_Session_Map (Live Configuration – Updated on Hot-Plug)

| Field | Type | Description | Example |
| --- | --- | --- | --- |
| slot_number | INTEGER PK | Physical slot 1–12 | 5 |
| pad_id | INTEGER FK | Currently connected pad (NULL if empty) | 125 or NULL |
| last_seen_timestamp | DATETIME | For hot-plug detection | 2025-12-15 10:23:45 |

### Table 3: Custom_Recordings

| Field | Type | Description | Example |
| --- | --- | --- | --- |
| recording_id | INTEGER PK | Unique ID | 45 |
| file_path | TEXT | Location on storage | "/audio/custom_45.mp3" |
| source | TEXT | "parent" or "grandparent" | "grandparent" |
| assigned_to_pad_id | INTEGER | Specific pad (nullable) | 125 or NULL |
| assigned_to_reward | TEXT | "correct_touch", "level_up", "general" | "correct_touch" |
| upload_timestamp | DATETIME | When received | 2025-12-10 14:30:00 |

### Table 4: Events_Log (Main Activity Table – Core of Statistics)

| Field | Type | Description | Example |
| --- | --- | --- | --- |
| event_id | INTEGER PK | Auto-increment | 15872 |
| timestamp | DATETIME | When event happened | 2025-12-15 10:45:33 |
| slot_number | INTEGER | Physical slot touched | 5 |
| pad_id | INTEGER | Pad type (from map) | 125 |
| touch_type | TEXT | tap / hold / repeated / swipe / multi | "hold" |
| intensity_level | TEXT | light / medium / hard | "medium" |
| peak_intensity_raw | INTEGER | Raw peak value (0–1023) | 612 |
| duration_ms | INTEGER | Length of touch | 1200 |
| interval_ms | INTEGER | Time since previous touch (for rhythm) | 450 |
| sequence_id | TEXT | If part of recognised sequence (future) | NULL or "combo_abc" |
| response_light_pattern | TEXT | chase / wave / flash / static | "wave" |
| response_led_colors | TEXT (JSON) | List of colors used | ["#00FF00", "#0000FF"] |
| response_vibration | TEXT | intensity + pattern | "70_pulse_3x" |
| sound_played_type | TEXT | system_tone / melody / speech / custom | "custom" |
| sound_id | INTEGER | Linked system or custom sound | 45 |
| session_id | INTEGER | Groups events in one play session | 892 |

### Table 5: Sessions (For Grouping Daily Activity)

| Field | Type | Description | Example |
| --- | --- | --- | --- |
| session_id | INTEGER PK | Auto-increment | 892 |
| start_timestamp | DATETIME | When session began (first touch after inactivity) | 2025-12-15 10:30:00 |
| end_timestamp | DATETIME | When session ended (last touch + timeout) | 2025-12-15 11:15:00 |
| play_level | INTEGER | Level active during session | 3 |
| play_mode | TEXT | free / follow_light / find_animal | "follow_light" |
| total_touches | INTEGER | Count (updated live) | 48 |

### Table 6: Settings (Current System Configuration)

| Field | Type | Description | Example |
| --- | --- | --- | --- |
| key | TEXT PK | Setting name | "current_level" |
| value | TEXT | Setting value | "3" |
| last_updated | DATETIME | When changed | 2025-12-14 20:15:00 |

## How Data Flows into Database

- Every touch → Event logged in Events_Log (with full classification fields)

- Session created/updated in Sessions table

- Pad plug/unplug → Current_Session_Map updated

- Custom audio received → new row in Custom_Recordings

- App requests stats → queries Events_Log + Sessions → returns summaries