LEVEL1: High-Level Overall Algorithm for TOKOTouch System (For Architecture Engineer)

Overall System Flow (in a few sentences)

The TOKOTouch system works as a continuous real-time feedback loop: SMART PADs continuously sample pressure from FSR sensors, average the readings, and buffer them. The CONTROL UNIT periodically polls all connected pads, collects the buffered touch data, classifies the touches, applies game logic (free play or guided tasks), decides on an appropriate response, and sends back commands for lights, vibration, and audio. The app can at any time change modes, levels, or create specific guided requests, and receives live updates and statistics. The system always prioritizes child safety and responsiveness.

Main Loops (High-Level)

- SMART PAD Loop (runs independently on each of up to 12 pads)

  - Continuous low-level sampling → averaging → buffering of FSR data with timestamps.

  - Waits for CONTROL UNIT to read buffer and clear it.

- CONTROL UNIT Main Loop (central brain on Raspberry Pi)

  - Polls pads regularly → receives touch buffers → classifies touches → makes decision (free play reward or guided task handling) → generates and sends output commands (lights/vib/sound) → logs events.

  - Runs parallel threads for app communication and error monitoring.

- App Interaction Loop

  - App sends commands (change level, set task, upload audio) → CONTROL UNIT updates state and request_response_map → sends live feedback and statistics back to app.

High-Level Architecture Layers

- Layer 1 – Data Acquisition: SMART PADs collect and buffer raw touch data.

- Layer 2 – Interpretation & Decision: CONTROL UNIT classifies touches and decides response using request_response_map (free_play rules vs active guided tasks).

- Layer 3 – Output & Feedback: CONTROL UNIT executes lights, vibration, and audio responses.

- Layer 4 – User Interface: TOKOApp controls modes/tasks and receives live stats.

This structure keeps the system modular: touch detection is mostly independent from response logic, and the request_response_map acts as the central configurable table linking user requests (free or guided) to specific outputs.

LEVEL 2: the detailed execution flow of the CONTROL UNIT algorithm

CONTROL UNIT – Detailed Execution Flow (Step by Step)

1. Startup Sequence (Executed Once)

- Initialize I²C / communication bus

- Scan all 12 slots → detect connected pads

- For each connected pad: read Pad_ID, LED count, vibrator presence → build pad_map

- Load current_level, current_mode (default = free_play), and volume from storage

- Load request_response_map from JSON file into memory

- Initialize audio system and logging

- Start two background threads:

  - Pad Polling Thread

  - App Communication Thread

- Set active_task = None

2. Pad Polling Thread (Runs continuously)

- Every PAD_POLL_INTERVAL seconds (e.g. 4–6 sec):

  - For each slot 1 to 12:

    - If pad exists in pad_map:

      - Send request to SMART PAD → get FSR_PAD_PRESSURE buffer (up to 10 entries)

      - If data received and not empty:

        - Call Process_Touch_Buffer(buffer, slot)

        - Send “clear buffer” command to the SMART PAD

      - Else if no response:

        - Retry once → if still fails, mark pad as disconnected and log error

3. Process_Touch_Buffer (Called from Polling)

- For each averaged entry in the received buffer:

  - If pressure (left or right) > threshold:

    - Calculate: peak, duration, balance, touch_type (tap / hold / press)

    - Create touch_event object (slot, pad_id, peak, duration, type, timestamp, etc.)

    - Send touch_event to Decision_Making()

4. Decision_Making (Core Logic – Called for every touch_event)

- IF current_mode == "free_play":

  - Search request_response_map["free_play"]["responses"] from highest to lowest priority

  - Find first matching response for this touch

  - Call Generate_Output(matching_response)

- ELSE (Guided Task mode):

  - If active_task exists:

    - Check if touch_event matches active_task.expected_pattern

      - Yes → Generate_Output(active_task.success_response) + clear active_task

      - No but valid touch → Generate_Output(active_task.fail_response)

    - If timeout reached → Generate_Output(active_task.hint_response)

- Log the touch_event + chosen response

5. Generate_Output (Response Execution)

- If response contains lights:

  - Determine which pads to light

  - Send LED commands to relevant SMART PADs (pattern, color, brightness)

- If response contains vibration:

  - Send vibration command to correct slot(s)

- If response contains sound:

  - Play corresponding sound (default or custom) at current volume

- Optional: queue timing between lights, vibration and sound for better feel

6. App Communication Thread (Runs in parallel)

- Continuously listen for connection from TOKOApp (Bluetooth preferred)

- When command received:

  - set_level X → update current_level

  - set_mode + task_id → set current_mode, load and activate active_task from map

  - upload_custom_audio → save file and update request_response_map

- Send live updates to app:

  - Current pad_map, active task, recent touches, statistics

- Handle disconnect → auto-reconnect when phone returns

7. Logging & Statistics

- After every decision: write to log file (timestamp + touch details + response)

- Maintain in-memory counters (today’s touches, per pad stats, etc.)

- When app requests statistics → calculate summaries and send

8. Error Handling (Called whenever needed)

- Communication timeout with pad → retry → mark disconnected if persistent

- Guided task timeout → trigger hint response

- Critical error → safe shutdown or restart of affected threads

TOKO System — Design Overview

Progression Architecture & System States

# 1. Progression Tiers

The system guides children through three tiers of increasing complexity. Each tier builds on the previous one.

| Tier | Name | Description | Key Rule |
| --- | --- | --- | --- |
| Tier 1 | Free Play | Any touch on any pad triggers a reward. No wrong answers. Child discovers cause and effect. | No conditions — every touch succeeds. |
| Tier 2 | Leveled Free Play | Rules tighten gradually. No specific pad required — touch quality and patterns matter. | Thresholds on pressure, duration, pad count, and sequence. |
| Tier 3 | Guided / Themed | Specific pads required. Tasks defined per theme. CARD_ID drives logic. | Must press correct CARD_ID pad(s) per task. |

# 2. Tier 2 — Level Progression

All thresholds are parameters — configurable per level or per parent preference. Nothing is hardcoded.

| Level | Name | Condition to Earn Reward | Notes |
| --- | --- | --- | --- |
| L1 | Any touch | Any touch on any pad | Entry point — no wrong answer |
| L2 | Pressure | Touch exceeds minimum pressure threshold | Threshold = configurable parameter |
| L3 | Pressure + duration | Touch meets both pressure AND duration minimums | Both thresholds = configurable parameters |
| L4 | Exploration | Child touches N different pads per session | N = configurable parameter |
| L5 | Sequence | Child touches any sequence of N pads | N = configurable parameter |
| L6 | Balance | Difference between FSR0 and FSR1 within threshold | Uses left/right sensor balance per pad |

# 3. Tier 3 — Guided Themes

Each theme is a list of tasks. Each task defines: expected CARD_ID(s), success response, fail response, and hint response. Physical slot position is irrelevant — CARD_ID is read from hardware at startup.

| Theme | Type | Notes |
| --- | --- | --- |
| Shapes | Built-in (default) | Ships with device. Basic geometric forms. |
| Animals | Built-in | Available as add-on theme pack. |
| Sports | Built-in | Available as add-on theme pack. |
| Colors | Built-in | Available as add-on theme pack. |
| Numbers | Built-in | Available as add-on theme pack. |
| Custom | Parent-created (via app) | Parent defines tasks and CARD_ID targets via app. |

# 4. Pad Connection States

Each SMART_PAD in the pad_map carries one of four connection states at any time.

| State | Trigger | System Behaviour |
| --- | --- | --- |
| Active | Responding normally | Normal operation. No alerts. |
| Disconnected — expected | Was in pad_map; now not responding | Likely magnetically detached. Alert parent if task-relevant. |
| Disconnected — unknown | Never responded since startup | Possibly not physically present. Report to parent. |
| Reconnected | Returns after disconnect | Auto-reintegrated silently. Quiet notification to parent. |

# 5. Missing Pad — Parent Alert Logic

## At Startup

If a task-required CARD_ID is missing from the scan, the system alerts the parent with three options:

- Wait — parent will go physically attach the missing pad

- Skip — continue the session without that pad

- Replace — put a different pad in that position

## Mid-Session

If a pad detaches during a session:

- Immediate pause + parent alert: Task-relevant pad detaches

- Log silently; include in end-of-session summary: Non-task pad detaches

- Auto-reintegrated; quiet notification only: Pad reconnects (magnetic reattachment)

# 6. Advancement Criteria

The system tracks child performance and suggests advancement to the next level. The parent makes the final decision via the app.

| Aspect | Definition |
| --- | --- |
| Default rule | X successful touches in the last Y sessions. Both X and Y are configurable parameters. |
| Scope | Parameters can be set per level or per parent preference. |
| Who decides | System detects readiness and suggests. Parent confirms via app. |
| Future-ready | Rule type is also a parameter — can be swapped later (e.g. consistency over time) without restructuring. |

# 7. Key Design Principles

- Everything parametric — no hardcoded thresholds, counts, or rules

- CARD_ID is identity — pad slot position is irrelevant to logic

- Parent-guided, system-informed — system suggests, parent decides

- One child per device — no profile switching needed

- Graceful degradation — missing pads never silently fail; parent is always informed

- Modular themes — built-in and custom themes use identical data structures