CONTROL UNIT algorithm

1. Initialization & Startup Sequence

- Runs once on power-on or reboot.

- Discovers which pads are connected.

- Loads configurations, defaults, and the request_response_map.

- Starts background threads for polling and app sync.

Pseudocode bullets:

- Initialize I²C bus

- For each slot 1 to 12:

  - Check if pad connected

  - If yes: read pad config (ID, LEDs, vibrator)

  - Store in pad_map[slot]

- Load current_level, current_mode from storage (default 1, "free_play")

- Load REQUEST_RESPONSE_MAP from file

- Start Pad_Polling_Thread

- Start App_Sync_Thread

- Initialize speaker and volume

2. Pad Polling & Data Collection (Background Thread)

- Polls every PAD_POLL_INTERVAL seconds (e.g. 5s)

- Asks each pad for its FSR buffer

- Processes data → clears buffer on pad

- Handles missing pads

Pseudocode bullets:

- While true:

  - For each slot in pad_map:

    - If slot has pad:

      - Request FSR_PAD_PRESSURE buffer from pad

      - If success:

        - Send buffer to touch classification

        - Send clear_buffer command to pad

      - Else:

        - Call Handle_Error(slot, "poll_fail")

  - Delay PAD_POLL_INTERVAL seconds

3. Touch Detection & Classification (Called from Polling)

- Analyzes buffer entries independently of modes

- Detects if pressure means a touch

- Classifies basic type (tap, hold, etc.)

Pseudocode bullets:

- For each entry in received buffer:

  - If avg_fsrl > THRESHOLD OR avg_fsrr > THRESHOLD:

    - peak = max(avg_fsrl, avg_fsrr)

    - duration ≈ current_time - entry.timestamp

    - type = "tap" if duration < 500ms else "hold" else "press"

    - balance = avg_fsrl - avg_fsrr

    - Create touch_event = {slot, pad_id, type, peak, duration, balance, timestamp}

    - Send touch_event to Decision_Making

4. Game/Mode Logic & Decision Making (Called from Classification)

- Decides what to do based on current mode

- Free play → always reward

- Guided task → check if touch matches request

Pseudocode bullets:

- If current_mode == "free_play":

  - Find matching response in REQUEST_RESPONSE_MAP["free_play"]["responses"]

  - Pick highest priority match

  - If found: send to Generate_Output

- Else if active_task exists:

  - If touch matches task.expected_pattern:

    - response = task.success_response

    - Clear active_task

  - Else if touch valid but wrong:

    - response = task.fail_response

  - Else if task timeout:

    - response = task.hint_response

  - If response: send to Generate_Output

- Log event + response

5. Output Generation & Sequencing

- Creates and sends commands for lights/vib/sound

- Handles multi-pad effects

Pseudocode bullets:

- If response.lights:

  - For each pad in response:

    - Send Light_Command(pad, pattern, color, brightness)

- If response.vibration:

  - Send Vibration_Command(slot, intensity, duration, pattern)

- If response.sound:

  - Play sound file or custom (with volume)

- Time commands to avoid overlap (e.g., queue)

6. App Communication & State Sync (Background Thread)

- Listens for app commands

- Sends live updates

- Handles connect/disconnect

Pseudocode bullets:

- While true:

  - If app_connected:

    - Receive command from app (e.g. set_level, set_task, upload_audio)

    - If set_level: current_level = value

    - If set_mode: current_mode = value

    - If set_task: active_task = map[task_id], active = true

    - If upload_audio: save file, update map

    - Send ack + current state (pad_map, level, mode, task)

  - Else:

    - Try reconnect

  - Delay 1 second or use event trigger

7. Logging & Statistics

- Records events

- Prepares summaries for app

Pseudocode bullets:

- On every touch/response:

  - Append to log: timestamp, event details, response

  - Update counters: total_touches, per_pad_touches

- When app asks for stats:

  - Compute summaries (daily touches, progress)

  - Send to app

8. Error Handling & Safety

- Catches problems

- Keeps system stable

Pseudocode bullets:

- On poll fail:

  - Retry 3 times

  - If still fail: mark pad disconnected, log error

- On timeout in guided task:

  - Trigger hint response

- If watchdog expires:

  - Restart system safely

This bullet-point format keeps everything clear and easy to analyze.