Pseudocode for TOKO_TOUCH SMART_PAD Input Algorithm

Below is the pseudocode for the SMART_PAD touch input processing. It covers:

- Calibration at power-on.

- Continuous raw sampling every FSR_RAW_SAMPLING_RATE ms (e.g., 50 ms).

- Averaging over FSR_AVG_SAMPLING_RATE ms (e.g., 500 ms) by accumulating raw samples and computing average.

- Subtracting calibration values.

- Storing in a FIFO buffer (vector) called FSR_PAD_PRESSURE with 10 entries (each entry: averaged FSRL, averaged FSRR, timestamp).

- Buffer management: New values pushed, oldest popped if full and not read; read values erased on request from CONTROL UNIT.

- CONTROL UNIT polling every few seconds (e.g., PAD_POLL_INTERVAL seconds) to read and clear buffer.

- This is written in a simple IF-THEN pseudocode style for clarity. Assume this runs on the SMART_PAD MCU (e.g., in C or Arduino-like environment). Each SMART_PAD (up to 12) runs this independently.

Constants

FSR_RAW_SAMPLING_RATE = 50  // ms between raw FSR reads

FSR_AVG_SAMPLING_RATE = 500  // ms over which to average (e.g., 10 raw samples if rate=50ms)

NUM_RAW_SAMPLES_FOR_AVG = FSR_AVG_SAMPLING_RATE / FSR_RAW_SAMPLING_RATE  // e.g., 10

BUFFER_SIZE = 10  // Number of averaged entries in FIFO vector

PAD_POLL_INTERVAL = 5  // seconds; CONTROL UNIT polls buffer this often (adjust as needed)

THRESHOLD_CALIBRATION_SAMPLES = 20  // Number of raw reads for initial calibration

Variables (on SMART_PAD)

calib_fsrl = 0  // Calibration baseline for left FSR

calib_fsrr = 0  // Calibration baseline for right FSR

temp_fsrl_sum = 0  // Temporary sum for left FSR averaging

temp_fsrr_sum = 0  // Temporary sum for right FSR averaging

raw_sample_count = 0  // Counter for raw samples in current average

fsr_pad_pressure = []  // FIFO vector (array/queue) of size BUFFER_SIZE; each entry: {avg_fsrl, avg_fsrr, timestamp_ms}

last_timestamp = 0  // Current system time in ms (e.g., from millis())

SMART_PAD Main Algorithm (Runs on Power-On and Loops Forever)

// Calibration Function (Called Once at Power-On)

PROCEDURE Calibrate_FSR()

SET calib_fsrl_sum = 0

SET calib_fsrr_sum = 0

FOR i = 1 TO THRESHOLD_CALIBRATION_SAMPLES DO

READ raw_fsrl = Analog_Read(FSRL_PIN)  // Raw ADC value (0-1023)

READ raw_fsrr = Analog_Read(FSRR_PIN)

ADD raw_fsrl TO calib_fsrl_sum

ADD raw_fsrr TO calib_fsrr_sum

DELAY FSR_RAW_SAMPLING_RATE ms

END FOR

SET calib_fsrl = calib_fsrl_sum / THRESHOLD_CALIBRATION_SAMPLES

SET calib_fsrr = calib_fsrr_sum / THRESHOLD_CALIBRATION_SAMPLES

END PROCEDURE

// Main Loop (After Calibration)

CALL Calibrate_FSR()  // Run once

WHILE TRUE DO  // Continuous loop

// Raw Sampling Step

READ raw_fsrl = Analog_Read(FSRL_PIN)

READ raw_fsrr = Analog_Read(FSRR_PIN)

ADD (raw_fsrl - calib_fsrl) TO temp_fsrl_sum  // Subtract calibration immediately

ADD (raw_fsrr - calib_fsrr) TO temp_fsrr_sum

INCREMENT raw_sample_count

DELAY FSR_RAW_SAMPLING_RATE ms  // Wait for next raw read

// Check if Ready to Average

IF raw_sample_count == NUM_RAW_SAMPLES_FOR_AVG THEN

SET avg_fsrl = temp_fsrl_sum / NUM_RAW_SAMPLES_FOR_AVG

SET avg_fsrr = temp_fsrr_sum / NUM_RAW_SAMPLES_FOR_AVG

SET current_timestamp = Get_Current_Time_ms()  // e.g., millis()

// Buffer Management (FIFO Push)

PUSH new_entry = {avg_fsrl, avg_fsrr, current_timestamp} TO fsr_pad_pressure

IF LENGTH(fsr_pad_pressure) > BUFFER_SIZE THEN

POP oldest_entry FROM fsr_pad_pressure  // Discard oldest to keep size 10

END IF

SET last_timestamp = current_timestamp  // Update last timestamp (for CONTROL UNIT reference)

// Reset for Next Average

SET temp_fsrl_sum = 0

SET temp_fsrr_sum = 0

SET raw_sample_count = 0

END IF

END WHILE

CONTROL UNIT Polling Algorithm (Runs on Raspberry Pi – Separate Thread or Loop)

// This polls each SMART_PAD (up to 12) independently every PAD_POLL_INTERVAL seconds

PROCEDURE Poll_Smart_Pad(pad_slot)  // e.g., slot 1-12

WHILE TRUE DO

DELAY PAD_POLL_INTERVAL seconds  // e.g., 5 sec

// Read Buffer from SMART_PAD via I2C/Bus

REQUEST fsr_pad_pressure FROM SMART_PAD at pad_slot  // Get full vector

IF request_success THEN

PROCESS the vector  // e.g., analyze pressures, update modes in app

SEND clear_command TO SMART_PAD at pad_slot  // Erase read values

// On SMART_PAD Side (Response to Clear Command)

IF clear_command_received THEN

CLEAR fsr_pad_pressure  // Erase all entries (or pop all)

END IF

ELSE

// Handle error (e.g., pad disconnected); buffer on pad will continue pushing new values

END IF

END WHILE

END PROCEDURE